FABRICATION ORDER SHEET

Enclosure & Stand

| Item Description | Custom HDPE Bin Enclosure & Aluminum Mounting Stand |
| --- | --- |
| Units | All dimensions are in centimeters (cm) unless specified otherwise. |
| Tolerances | Sheet cutting ± 0.5 mm; hole centerlines ± 1.0 mm. |

1. Bin Enclosure — Outer Shell (Material: 5 mm HDPE Sheet)

| Overall Assembled Size | 20.0 cm Wide × 18.0 cm Deep × 30.5 cm High |
| --- | --- |
| Material | 5 mm High-Density Polyethylene (HDPE) Sheet (Matte White or Grey) |
| Assembly Method | Hot-air / extrusion plastic welding on static joints; stainless steel hinges on movable panels. |

Panel Cut List & Machining

| Item # | Part Name | Qty | Finished Size (cm) | Machining & Assembly Instructions |
| --- | --- | --- | --- | --- |
| 1.1 | Base Panel | 1 | 20.0 × 18.0 | Flat bottom plate. Left and Right side panels weld on top of this plate flush with outer edges. The 20.0 cm × 18.0 cm Base Panel defines the overall outside footprint. |
| 1.2 | Left Side Wall | 1 | 18.0 × 30.5 | Full depth wall (18.0 cm). Weld bottom edge to Base Panel (Item 1.1). |
| 1.3 | Right Side Wall | 1 | 18.0 × 30.5 | Full depth wall (18.0 cm). Weld bottom edge to Base Panel (Item 1.1). |
| 1.4 | Back Wall | 1 | 19.0 × 30.5 | Fits between Left and Right walls; the 19.0 cm dimension is the clear width between the 5 mm thick side walls. Weld full height along both vertical sides and bottom to Base. |
| 1.5 | Upper Front Panel | 1 | 19.0 × 17.5 | Fixed / Welded. Fits between Left and Right walls; the 19.0 cm dimension is the clear width between the 5 mm thick side walls. Covers top 17.5 cm of front; the 19.0 cm dimension is the clear width between the 5 mm thick side walls. Drill 2× Ø16 mm through-holes centered horizontally, 3 cm from bottom edge of this panel. |
| 1.6 | Top Lid | 1 | 20.0 × 18.0 | Hinged (Do NOT weld). Attached to top edge of Back Wall using 2× small stainless steel butt hinges. Lid rests on top of the side walls as a cap. |
| 1.7 | Lower Access Door | 1 | 18.8 × 12.8 | Hinged Door. Fits lower front opening with 1–2 mm perimeter clearance on all four sides. Fabricator to form the opening to suit the stated finished door size and clearance. Attach 2× small hinges on left edge. Install strike plate for magnetic push-catch on right inside edge. |
| 1.8 | Internal Divider | 1 | 16.0 × 13.0 | Vertical divider plate. Center-weld vertically inside the lower chamber to divide the bottom bay into two equal compartments. Orientation: 13.0 cm vertical height × 16.0 cm depth. |

Assembly & Weld Schematic

ASSEMBLY & WELD SCHEMATIC

                     ┌── Top Lid (20 x 18 cm) [HINGED AT REAR]
                     │
        ┌────────────┴────────────┐
        │ Upper Front Panel        │
        │ (19 x 17.5 cm)           │  ◀── WELD to Left & Right Walls
        │ [o  o] (2x 16mm holes)  │
        ╞═══════════════════════════╡
[HINGE] │ Lower Access Door        │
        │ (18.8 x 12.8 cm)         │  ◀── HINGED on Left (Swings Open)
        │                    [LATCH]│
        └───────────────────────────┘
          ▲                       ▲
          │                       │
     Left Wall              Right Wall       Back Wall (19 x 30.5 cm)
   (18 x 30.5 cm)          (18 x 30.5 cm)     WELDED at rear

2. Internal Mechanical Parts (Flap & Removable Caddies)

| Item # | Part Name | Qty | Finished Size (cm) | Material | Fabrication Instructions |
| --- | --- | --- | --- | --- | --- |
| 2.1 | Tilting Flap | 1 | 14.0 × 12.0 | 3 mm PTFE (Teflon) or 3 mm HDPE | Flat sheet. Smooth and deburr all 4 edges. Corner radii: 5 mm rounded corners. Central pivot shaft along centerline (front-to-back) for bi-directional ±40° tilt. |
| 2.2 | Dry Waste Caddy | 1 | 7.0 (W) × 15.0 (D) × 12.5 (H) | 2–3 mm HDPE | Folded/welded 5-sided box (open top). Dimensions stated are finished external dimensions. Include a 10 mm folded lip/handle on the front top edge for pull-out extraction. |
| 2.3 | Wet Waste Caddy | 1 | 7.0 (W) × 15.0 (D) × 12.5 (H) | 2–3 mm HDPE | Folded/welded 5-sided box (open top). Dimensions stated are finished external dimensions. All bottom and corner seams 100% continuously welded to be completely water-tight / leak-proof. Include front pull-out lip handle. Water-tightness to be verified by a basic water-fill leak test before delivery. |

3. Stand & Base Plate Assembly

| Item # | Part Name | Qty | Dimensions / Spec | Material | Fabrication Instructions |
| --- | --- | --- | --- | --- | --- |
| 3.1 | Support Pole | 1 | OD: 50.0 mm • Length: 125.0 cm (1250 mm) • Wall: minimum 2.5 mm | Aluminum Round Pipe (6061 or 6063) | Cut square on both ends. Deburr inner and outer edges. Drill 1× Ø15 mm grommet hole 5 cm from top (cable entry). Drill 1× Ø15 mm grommet hole 5 cm from bottom (cable exit). |
| 3.2 | Base Flange | 1 | To fit 50.0 mm OD pipe; socket/collar internal diameter to suit the selected pipe with a secure set-screw fit. | Cast Iron or Cast Aluminum | Circular floor flange / socket collar with 4× M8 bolt holes. Pipe slides into socket and locks via set screws. |
| 3.3 | Base Plate | 1 | 40.0 × 40.0 cm, Thickness: 19 mm | Marine-Grade Plywood, 19 mm thick | Square base plate. Pre-drill 4× Ø9 mm through-holes matching the bolt pattern of the Base Flange (Item 3.2). Counterbore bottom side so M8 bolt heads sit flush. |

4. Hardware & Fasteners Required (To be supplied / assembled)

| Item # | Description | Qty | Where Used |
| --- | --- | --- | --- |
| 4.1 | RS PRO 304 Stainless Steel Butt Hinges, 38 × 40 × 2 mm | Reference: RS PRO Stock No. 7491706. Stainless steel 304; free-swinging; 38 × 40 × 2 mm. | Qty 4 total: 2 for Top Lid (Item 1.6); 2 for Access Door (Item 1.7). Vendor may procure this exact hinge or an equivalent after approval. |
| 4.2 | Magnetic Push-to-Open Touch Latch with strike plate | 1 | Installed on inside right wall to latch Access Door (Item 1.7). Compact mechanical cabinet type. |
| 4.3 | M8 × 45 mm Hex Through-Bolts + Fender Washers + Nyloc Nuts | 4 | Fastens Base Flange (Item 3.2) through Base Plate (Item 3.3) |
| 4.4 | Stainless Steel M3.5 Self-Tapping Screws (10 mm) | 16 | For mounting hinges and magnetic latch to HDPE panels |

| ☐ | All HDPE cuts are square, burr-free, and clean. |
| --- | --- |
| ☐ | Upper front panel (19 × 17.5 cm) is welded fixed; lower door (18.8 × 12.8 cm) is mounted on hinges with clearance to swing freely. |
| ☐ | Bottom seams of Item 2.3 (Wet Caddy) are continuously welded and tested water-tight. |
| ☐ | Aluminum pole cut to 125 cm with both cable pass-through holes deburred. |
| ☐ | Base plate pre-drilled to match the floor flange bolt circle. |

| Fabrication Clarification | Requirement |
| --- | --- |
| Panel fit / dimensions | All enclosure panel dimensions are finished sizes. Side walls are 5 mm thick. Panels specified as fitting between the side walls use the stated clear width. |
| Lower access opening | Provide 1–2 mm perimeter clearance around the 18.8 × 12.8 cm finished access door. |
| Tilting flap | Central pivot shaft along centerline (front-to-back) for bi-directional ±40° tilt. |
| Support pole | 50.0 mm OD aluminum round pipe, minimum 2.5 mm wall, 1250 mm long. |
| Base flange | Flange/socket specifically sized for 50.0 mm OD pipe. Four M8 holes; bolt pattern to be matched to Base Plate. |
| Base plate | 19 mm marine-grade plywood, 40.0 × 40.0 cm. Counterbores on underside shall suit M8 bolt heads and leave them flush. |
| Caddies | Caddy dimensions are finished external dimensions. |
| Wet caddy test | After welding, perform a basic water-fill leak test to confirm water-tight / leak-proof construction. |

Recommended Procurement References

Hinge reference: RS PRO 304 Stainless Steel Butt Hinge, 38 × 40 × 2 mm (Stock No. 7491706)

The following are reference products/specifications only. Vendor may supply equivalent components, subject to dimensional compatibility and approval. The Base Flange is intentionally not linked to a specific off-the-shelf product; vendor must propose a compatible flange for the specified 50.0 mm OD pipe.

| Item | Component | Reference / Requirement | Vendor Action |
| --- | --- | --- | --- |
| 4.2 | Magnetic Push-to-Open Touch Latch with strike plate | Compact mechanical cabinet push-to-open magnetic latch with matching strike plate. | Installed on inside right wall to latch Access Door (Item 1.7). Compact mechanical cabinet type. |
| 3.2 | Base flange | No fixed product link. Must fit 50.0 mm OD aluminum pipe, have set-screw locking and 4 × M8 mounting holes. | Vendor to propose exact flange/socket and provide dimensions/photo for approval before purchase. |

5. Summary Checklist for Vendor

Magnetic latch: Use a compact mechanical push-to-open magnetic cabinet latch with matching strike plate. A specific model is not mandated because the vendor should verify fit to the 5 mm HDPE door.